[image: image1.png]2528320 "
SER7AB MR

KF 4}

y 2R

A Raiin,
B TR e Date code 7 ... = March 1967





[image: image2.png]



[image: image3.png]


ICCA – Engine Dress Review –Section # 11 (V2 – V 1)

1. Engine Mounts 
Engine mount 

Factory installed engine mounts are rubber cushions vulcanized to a plain steel backing.  All motor mounts have a date code molded into the mount. 

Wedge engines – The mounts are secured to the block with zinc plated hex head bolts, loose washers and nuts.  The mounts are secured to the K member with Hex head silver zinc plated bolts with loose washers for wedge motors only.  

Hemi engines -   The mounts are secured to the block with zinc plated hex nuts and loose washers that thread onto the studs that are in the side of the block.  The left hand mount is secured to the K member with Hex head silver zinc plated bolts with a loose washer.  The right hand mount is secured to the K member with a special Marsdin type flanged nut.  The mount was installed onto the engine prior to paint.  The complete mount should be painted engine color.   

Engine mount isolator reference

[image: image4.png]»a ﬁ\%/ ﬁ‘.\ﬁ\. i




[image: image5.png]



	Motor Mount Date Codes

	Year
	Dots
	Month

	1967
	. . .
	March

	1969
	. . . .
	[image: image6.png]


April

	1971
	. . . . .
	May


Engine mount brackets

All engine motor mount brackets are painted semi-gloss black. The exception is the 426 Hemi engine which had the motor mount bracket and isolator mounted to the engine prior to paint.
[image: image7.png]



[image: image8.png]



2. Valve Cover Related Components

PVC Breathers

All PVC breathers that were available for 1966-78 were stamped steel and painted semi-gloss black. The hose that goes from the main nipple to the air cleaner was a black ribbed rubber hose. There was a black zinc plated spring clamp installed to secure the hose to the breather as well as the air cleaner.  

· Single nipple breather – was the standard breather used on all big block, small block Hemi’s and Six pack engines. 

· Dual nipple breather - was used on 1970 big blocks, small blocks, Hemi's, 6 pack engines used them with the fuel vapor emissions systems
· Triple nipple breather - was used on 1971 big blocks, small blocks, Hemi's, 6 pack' engines used them with the fuel vapor emissions systems
[image: image9.png]


[image: image10.png]VRO
’0&)‘0"2»}\{@}2}%‘@)‘0"0 0:6’ oy !
\/j{ \/\/ ‘/ “ %) .. A A




[image: image11.png]



[image: image12.png]


[image: image13.png]



[image: image14.png]



[image: image15.png]E}L Ny




[image: image16.png]



PCV Valves
All engines had a PVC valve.   The following chart will indicate the type and configuration. 

	Ref 
	Year
	Engine
	Type
	Comments

	A
	1962-69
	6 cyl and wedge V8’s
	Metal elbow
	Painted along with valve covers
Part # 2808622

	A
	1964-67
	Hemi
	Metal elbow 
	It was left natural

	A
	1965-67
	272 4 bbl
	Metal elbow
	It was left natural

	C or D
	1970 -72
	6 cyl and small block
	Barrel type
	Painted engine color

	B or D
	1970-72
	B/RB 
	Three ridge type
	Painted engine color. 3671076

	B
	1970-72
	Hemi
	Three ridge type
	It was left natural. 3671076

	C
	1973-75
	All
	Plastic elbow
	Part number cast into the body

	D
	1970-72
	B/RB 
	No ribs
	Painted engine color. 3671076


[image: image17.png]



Oil fill caps 

All oil fill caps are stamped steel

· Wedge engines - the caps are painted engine color.  There are at least two style caps that have been discovered.  

· 426 Hemi engines - the fill cap has a satin chrome finished cap.  



3. Air Cleaner

Air cleaner assembly and element

Air cleaners cover 

340 – 4 bbl (E55 code) – E body
The cover is stamped steel painted semi-gloss black. On the bottom outer edge there is a cork composite seal (Ref part # 2536016). 
383 – 4 bbl (E63 code) and 440 – 4 bbl (E86 code)

The cover is stamped steel for E63 and E86 codes were identical and painted semi-gloss black.  On the bottom outer edge there is a cork composite seal (Ref part # 2536016).

440-6 bbl (E86 code) and 426 Hemi (E74 code)

The covers is stamped steel for the E63 and E74 were identical and painted a shade of orange just slightly lighter then the engine color. These lids were provided by Fram.
Filter Element

340-4, 383-4 and 440 -4 - The filter element is round was a white-to-yellowish corrugated paper with an expanded plated steel band sealed in molded orange rubber. The expanded metal had a series of round holes. The screen on the back side of the paper is a square mesh   

1968 426 Hemi - The filter element is round (part # 2658856) was a white-to-yellowish corrugated paper with an expanded plated steel band sealed in molded orange rubber. The expanded metal had a series of round holes. The screen on the back side of the paper is a square mesh   

1970 440+6 and 426 Hemi - The filter element is oval shaped was a white-to-yellowish corrugated paper with an expanded plated steel band sealed in molded orange rubber. The expanded metal has a series of round holes. There was a series of “D” shaped holes on the original filters.  The screen on the back side of the paper is a square mesh.  





�





�





�





�














1971 Three nipple breather





Two nipple breather





Single nipple breather 1970











PCV Reference 





�





�





�





�





1970 426 Hemi engine – Satin chrome plated. Reproduction the transition from the large to the small diameter is too long.





Style B 1970 383 or 440





Style A 1970 383 or 440





1970 340 





























Original oval air filter 





�





















































�



























































































































































































































































�





1968-72 big and small block brackets












































































































































�





October 1969 date code
































�








Original round air filter - 4 bbl engine part # 2402677. 1968 Hemi Engine is part # 2658856 There is 6 rows of holes and 2 rows of half circles.











Reproduction oval air filter has a diagonal mesh on the inside of the element





�




















�





Single nipple breather 1968 and 1969





Chrome = High performance cars


Painted black = Low performance cars







































































�





























�




















































































































D




































































PAGE  
# 11 -1-1
                                                                                                                     © ICCA/MMCDetroit 2008

